U.S. DEPARTMENT OF COMMERCE

NATIONAL OCEANIC AND ATMOSPHERIC ADMINISTRATION

NATIONAL WEATHER SERVICE

NATIONAL METEOROLOGICAL CENTER

Office Note 388

G  R  I  B

(Edition 1)

THE WMO FORMAT

FOR

THE STORAGE OF WEATHER PRODUCT INFORMATION

AND

THE EXCHANGE OF WEATHER PRODUCT MESSAGES

IN GRIDDED BINARY FORM

John D. Stackpole

Automation Division

Revised
(see overleaf)

July 31, 2026

This is an unreviewed manuscript,
primarily intended for informal exchange of information
among NMC staff members

REVISION HISTORY

since the last full revision/reprinting

dated March 3, 1993

August 31, 1993§	p.6:	Added orig center for NWS/NMC/Reanal
	and corrected DeBilt number.
p.7:	Added gen code 70 for QLM hurricane
p.9:	Added grids 75,76,77 for QLM
p.13,14: Reoriented Grids 37-44 (WAFC/ICAO)
	such that left hand column is at
	330E (30W)
p.13,13.1:  Added table of number of points in
	lat circles of grids 37-44
p.15:	Added description of Grid 1 
p.18:	Corrected long grid spacing in degrees
	and corrected location of Map 204
p.20	Adjusted location of map 208 to
	conform to change in map 204
p.28:	Added MSL variants 128 & 129; Added
	lat. lon. as parameters 176 & 177; 	added parameters 204, 205, 211, 212,
	218.
p.29:	Added Note 4
p.48:	Clarification of table 11.

Appendix A.  Revised to include "Z" as first character in WMO header.  Used for "off-hour" forecast hours.

GRIB Edition 1

INTRODUCTION

The World Meteorological Organization (WMO) Commission for Basic Systems (CBS) Extraordinary Meeting Number VIII (1985) approved a general purpose, bit-oriented data exchange format, designated FM 92-VIII Ext. GRIB (GRIdded Binary).  It is an efficient vehicle for transmitting large volumes of gridded data to automated centers over high speed telecommunication lines using modern protocols.  By packing information into the GRIB code, messages can be made more compact than character oriented bulletins, producing faster computer-to-computer transmissions.  GRIB can equally well serve as a data storage format, generating the same efficiencies relative to information storage and retrieval devices.

Changes and extensions to GRIB were approved at the regular meeting of the WMO/CBS in February, 1988; additional changes were introduced at the CBS/WGDM/Sub-Group on Data Representation meetings in May 1989 and in October 1990.  The last set of changes were of such structural magnitude as to require a new Edition of GRIB, Edition 1, which this document describes.

Note: the Edition number is in the same location, relative to the start of the GRIB message, for all Editions. Thus, decoding programs can detect which Edition layout to expect in the remaining portion of the message.  This is appropriate for archives of messages encoded in earlier Editions or during transition periods.

Each GRIB record intended for either transmission or storage contains single value, or a matrix of values, at an array of grid points, or a set of spectral coefficients, for parameters at a single level (or layer) as a continuous bit stream.  Logical divisions of the record are designated as "sections", each of which provides control information and/or data.  A GRIB record consists of six sections, two of which are optional:

(0) Indicator Section

(1) Product Definition Section (PDS)

(2) Grid Description Section (GDS)  -  optional

(3) Bit Map Section (BMS)  		 -  optional

(4) Binary Data Section (BDS)

(5) '7777' (ASCII Characters)

Most centers require bulletin headers to enable them to receive, identify, and switch messages; NMC is no exception.  The standard WMO abbreviated heading for GRIB is described in Appendix A.

In this documentation, certain symbols are used to clarify the contents of octets (groups of eight consecutive binary bits, "bytes" in American usage).  If unadorned letters are used, they are symbolic and their meanings are described in the text; a decimal number is simply printed as is; a character or string of characters is represented inside single quote marks.  International Alphabet No. 5, which is identical in its essential elements to the U.S. National Standard 7-bit ASCII, is used for character representation in the GRIB code.

Octets are numbered consecutively from the start of each section; bits within an octet are also numbered from left (the most significant bit) to right (the least significant bit).  Thus an octet with bit 8 set to the value 1 would have the integer value 1; bit 7 set to one would have a value of 2, etc.

The numbering of Tables in the following text corresponds to the description of GRIB in the WMO Manual on Codes<>.  Some additional tables not found in the WMO Manual are indicated by letters.  These, generally, contain information unique to the NWS or NOAA.

A final caveat: this document does not include all the features in GRIB as described in the Manual on Codes.  This document selects out those features which are of particular interest to the National Weather Service at the present time.  Some of the "advanced" features are alluded to in this document; please refer to the Manual for full details.

DATA PACKING METHOD.

The code form represents numeric data as a series of binary digits (bits).  Such data representation is independent of any particular machine representation; by convention data lengths are measured in octets.  Data are coded as binary integers using the minimum number of bits required for the desired precision.  Numeric values, with units as shown in Table 2, are first scaled by a power of ten to achieve an appropriate decimal precision, a reference value is subtracted from them to reduce redundancy and eliminate negative values, and they are then further scaled by a power of two to pack them into a pre-selected word length.

The representation of a single value is such that:

Y * 10D = R + (X * 2E)

where

Y = original or unpacked value; units as in Table 2;

D = decimal scale factor, to achieve desired precision

(sign bit, followed by a 15-bit integer);

R = reference value (32 bits);

X = internal value (No. of bits varies for each record);

E = binary scale factor for word-length packing
		(sign bit, followed by a 15-bit integer).

The reference value (R) is the minimum value of the decimally scaled data that is being encoded.  R is placed in the Binary Data Section in four octets as a single precision floating-point number:

sAAAAAAA BBBBBBBB BBBBBBBB BBBBBBBB

where s = sign bit, encoded as   0 => positive

1 => negative

A = 7-bit binary integer, the characteristic

B = 24-bit binary integer, the mantissa.

The appropriate formula to recover the value of R is:

R = (-1)s * 2(-24) * B * 16(A-64)

This formula is the standard IBM representation for a single precision (real) floating point number.  (Consideration is being given to using the IEEE floating point representation in the future, in a later Edition of GRIB.)

Both the decimal scaling factor and/or the binary scaling factors may equal 0.  It is not necessary to use the binary scaling to "fit" the numbers into a prespecified word size; an alternative encoding practice is to change the word length (the same for all the points, of course) to accommodate the largest departure from the reference value.  A properly written GRIB decoder should be able to decode either option with no change in the logic as long as the decoder makes no prior assumptions about word lengths.  All the necessary parameters are included in the GRIB message, of course.

GRIB CODE FORM:

With the exception of the Indicator Section and the End Section all octets contain binary values.  All sections contain an even number of octets; the variable length sections are padded with zero values as necessary.  These extra bits must be accounted for in finding one's way through the sections; their content should be ignored.

section 0: the Indicator Section (IS)

The indicator section serves to identify the start of the record in a human readable form; to indicate the total length of the message, and to indicate the Edition number of the message.  It is always eight octets long.

Octet no.             IS Content

1-4	'GRIB'  (Coded CCITT-ITA No. 5) (ASCII);

	5-7	Total length, in octets, of GRIB message
			(including Sections 0 & 5);

	8	Edition number - currently  1

section 1: the PRODUCT DEFINITION SECTION (PDS).

The PDS contains indicators for the Parameter table Version, the originating center, the numerical model (or "generating process") that created the data, the geographical area covered by the data, the parameter itself, and the actual values for the appropriate vertical level or layer, the decimal scale factor, and date/time information.  The PDS is at least 28 octets long but it may be longer if an originating center chooses to make it so.  Users of GRIB messages are strongly urged to use the length-of-section portion of the PDS to determine where the next section begins.  Do not assume a fixed octet length.

Octet no.				PDS Content

1 - 3      Length in octets of the Product Definition Section

4         Parameter Table Version number.
	Currently Version 1 for international exchange
	Parameter table version numbers 128-254 are
	reserved for local use.

5         Identification of center (See Table 0)

6         Generating process ID number 
	(allocated by the originating center; See Table A)

7         Grid Identification (geographical location and
								area; See Table B)

8         Flag specifying the presence or absence of a GDS
		          or a BMS       (See Table 1)

9         Indicator of parameter and units (See Table 2)

10        Indicator of type of level or layer 
					(See Tables 3 & 3a)

11-12     Height, pressure, etc. of the level or layer
					(See Table 3)

13        Year of century     \  Initial (or Reference)

|  time of forecast - UTC

14        Month of year       |

|          or

15        Day of month         >

|  Start of time period

16        Hour of day         |    for averaging or

|    accumulation of

17        Minute of hour      /    analyses

18        Forecast time unit (see Table 4)

19        P1 - Period of time (Number of time units)
				(0 for analysis or initialized analysis.)
				Units of time given by content of octet 18.

20        P2 - Period of time (Number of time units)
					or
				Time interval between successive analyses,
				successive initialized analyses, or forecasts,
				undergoing averaging or accumulation.
				Units given by octet 18.

21        Time range indicator (See Table 5)

22-23     Number included in average, when octet 21
		 	   (Table 5) indicates an average or
			   accumulation; otherwise set to zero.

24		 Number Missing from averages or accumulations.

25		 Century of Initial (Reference) time
		 		(=20 until Jan. 1, 2001)

26		 Reserved - set to 0

27-28	The decimal scale factor D.  A negative value
	is indicated by setting the high order bit
	(bit No. 1) in octet 27 to 1 (on).

29-40	Reserved (need not be present)

41-...	Reserved for originating center use.

Note: Octet 8 may indicate the presence of the Grid Description Section (GDS) even though octet 7 specifies a predefined grid.  In this case the GDS must describe that grid - this device serves as a mechanism for transmitting new "predefined" grids to users prior to their formal publication in this or the official WMO documentation.

TABLES FOR THE PDS

TABLE 0.  ORIGINATING CENTER
(PDS Octet 5)

VALUE			CENTER

07	US Weather Service - National Met. Center
	08	US NWS - NMC - Reanalysis project.
	34	Japanese Meteorological Agency - Tokyo
	54	Canadian Meteorological Service - Montreal
	58	US Navy  - Fleet Numerical Oceanography Center
	59	NOAA Forecast Systems Lab, Boulder CO
	74	U.K. Met Office - Bracknell
	97	European Space Agency (ESA)
	98	European Center for Medium-Range Weather
		Forecasts - Reading
	99	DeBilt

	150	ABRFC - Arkansas-Red River RFC, Tulsa OK
	151	Alaska RFC, Anchorage, AK
	152	CBRFC - Colorado Basin RFC, Salt Lake City, UT
	153	CNRFC - California-Nevada RFC, Sacramento, CA
	154	LMRFC - Lower Mississippi RFC, Slidel, LA
	155	MARFC - Middle Atlantic RFC, State College, PA
	156	MBRFC - Missouri Basin RFC, Kansas City, MO
	157	NCRFC - North Central RFC, Minneapolis, MN
	158	NERFC - Northeast RFC, Hartford, CT
	159	NWRFC - Northwest RFC, Portland, OR
	160	OHRFC - Ohio Basin RFC, Cincinnati, OH
	161	SERFC - Southeast RFC, Atlanta, GA
	162	WGRFC - West Gulf RFC, Fort Worth, TX

	170	OUN   - Norman OK WFO

TABLE A.		Generating Process or Model
(PDS Octet 6)

VALUE		MODEL

10	Global Wind-Wave Forecast Model
	19	Limited-area Fine Mesh (LFM) analysis
	25	Snow Cover Analysis
	39	Nested Grid forecast Model (NGM)
	42	Global Optimum Interpolation Analysis (GOI)
		from "Aviation" run
	43	Global Optimum Interpolation Analysis (GOI)
		from "Final" run
	44	Sea Surface Temperature Analysis
	53	LFM-Fourth Order Forecast Model
	64	Regional Optimum Interpolation Analysis (ROI)
	68	80 wave triangular, 18-layer Spectral model
		from "Aviation" run
	69	80 wave triangular, 18 layer Spectral model
		from "Medium Range Forecast" run
	70	Quasi-Lagrangian Hurricane Model (QLM)
	73	Fog Forecast model - Ocean Prod. Center
	74	Gulf of Mexico Wind/Wave
	75	Gulf of Alaska Wind/Wave
	76	Bias corrected Medium Range Forecast	
	77	126 wave triangular, 18 layer Spectral model
		from "Aviation" run
	78	126 wave triangular, 18 layer Spectral model
		from "Medium Range Forecast" run
	79	Backup from the previous run
	80	62 wave triangular, 18 layer Spectral model
		from "Medium Range Forecast" run
	81	Spectral Statistical Interpolation (SSI)
		analysis from "Aviation" run.
	82	Spectral Statistical Interpolation (SSI)
		analysis from "Final" run.
	83	ETA Model - 80 km version
	84	ETA Model - 40 km version
	85	ETA Model - 30 km version
	86	MAPS Model, from Forecast Systems Lab
			(Isentropic; scale: 60km at 40N)
	150	NWS River Forecast System (NWSRFS)
	151	NWS Flash Flood Guidance System (NWSFFGS)
	152	WSR-88D Stage II Precipitation Analysis
	153	WSR-88D Stage III Precipitation Analysis

TABLE B.   GRID IDENTIFICATION
(PDS Octet 7)

MASTER LIST OF NMC STORAGE GRIDS

VALUE                GRID 						 GRID

INCREMENT

1	1679-point (73x23) Mercator grid with	5 degs of
		(1,1) at (0W,48.09S), (73,23) at (0W,	longitude
		48.09N); I increasing eastward, 
		Equator at J=12.

	2	10512-point (144x73) global longitude-	2.5 deg
		latitude grid.  (1,1) at 0E, 90N, 
		matrix layout.  N.B.: prime meridian 
		not duplicated.

	3	65160-point (360x181) global longitude-	1.0 deg
		latitude grid.  (1,1) at 0E, 90N, 
		matrix layout.  N.B.: prime meridian 
		not duplicated.
		
	5	3021-point (53x57) N. Hemisphere polar	190.5 km
		stereographic grid oriented 105W; Pole	at 60N
		at (27,49). (LFM analysis)

	6	2385-point (53x45) N. Hemisphere polar	190.5 km
		stereographic grid oriented 105W; Pole	at 60N
		at (27,49). (LFM Forecast)

	21-26	International Exchange and 
		Family of Services (FOS) grids - see below

  27         4225-point (65x65) N. Hemisphere polar   381 km
             stereographic grid oriented 80W; Pole    at 60N
             at (33,33).

  28         4225-point (65x65) S. Hemisphere polar   381 km
             stereographic grid oriented 100E; Pole   at 60S
             at (33,33).

  29         5365-point (145x37) N. Hemisphere lon-   2.5 degs
             gitude/latitude grid for latitudes 0N
             to 90N; (1,1) at (0E,0N).

  30         5365-point (145x37) S. Hemisphere lon-   2.5 degs
             gitude/latitude grid for latitudes 90S
             to 0S; (1,1) at (0E,90S).

33         8326-point (181x46) N. Hemisphere lon-   2 degs
             gitude/latitude grid for latitudes 0N
             to 90N; (1,1) at (0E,0N).

	34	8326-point (181x46) S. Hemisphere lon-	2 degs
		gitude/latitude grid for latitudes 90S
		to 0S; (1,1) at (0E,90S).

	37 - 44	Eight lat-long 1.25x1.25 "thinned" grids,
		covering the globe by octants of 3447 points.
		Full GRIB specifications below.
		For WAFC, ICAO, Family of Services (FOS),
		and International exchange.

	50	Family of Services "regional grid" - see below.

	55	6177-point (87x71) N. Hemisphere polar	254 km
		stereographic grid oriented 105W; Pole	at 60N
		at (44,38). (2/3 bedient NH sfc anl)

	56	6177-point (87x71) N. Hemisphere polar	127 km
		stereographic grid oriented 105W; Pole	at 60N
		at (40,73). (1/3 bedient NA sfc anl)

	61-64	International Exchange & FOS grids - see below.

	75	12321-point (111x111) N. Hemisphere	40 km at
		Lambert Conformal grid.  No fixed	30&60 deg N
		location; used by QLM Hurricane model.

	76	12321-point (111x111) S. Hemisphere	40 km at
		Lambert Conformal grid.  No fixed	30&60 deg S
		location; used by QLM Hurricane model.

	77	12321-point (111x111) N. Hemisphere	40 km at
		Mercator grid.  No fixed location;	22.5 deg N&S
		used by QLM Hurricane model.

	85	32400-point (360x90) N. Hemisphere	1 deg
		longitude/latitude grid;
		longitudes: 0.5E to 359.5E (0.5W); 
		latitudes: 0.5N to 89.5N; 
		origin (1,1) at (0.5E,0.5N)

	86	32400-point (360x90) S. Hemisphere	1 deg
		longitude/latitude grid;
		longitudes: 0.5E to 359.5E (0.5W); 
		latitudes: 89.5S to 0.5S; 
		origin (1,1) at (0.5E,89.5S)

	87	5022 point (81x62) N. Hemisphere  polar	68.153 km
		stereographic grid oriented at 105W.	at 60N
		Pole at (31.91, 112.53) Used for RUC.
		(60 km at 40N).  See below for GRIB
		specification.

	90	12902 point (92x141 semi-staggered)    lat.14/26 deg
		lat. long., rotated such that center   lon.15/26 deg
		located at 52.0N, 111.0W; LL at 37.5W, 35S
		Unfilled E grid for 80 km ETA model

	91	25803 point (183x141)                  lat.14/26 deg
		lat. long., rotated such that center   lon.15/26 deg
		located at 52.0N, 111.0W; LL at 37.5W,35S
		Filled E grid for 80 km ETA model

	92	2 4162 point (127x191 semi-staggered)  lat. 5/57 deg
		lat. long., rotated such that center   lon. 5/18 deg
		located at 41.0N, 97.0W; LL at 35W,25S
		Unfilled E grid for 40 km ETA model

	93	48323 point (253x191)lat. long.,       lat.15/57 deg
		rotated such that center located       lon.5/18 deg
		at 41.0N, 97.0W; LL at 35W ,25S
		Filled E grid for 40 km ETA model

	98	Global Gaussian T62 grid.  See GRIB 
		specifications below

 100         6889-point (83x83) N. Hemisphere polar   91.452 km
             stereographic grid oriented 105W; Pole   at 60N
             at (40.5,88.5). (NGM Original C-Grid)

	101	10283-point (113x91) N. Hemisphere	91.452 km
		polar stereographic grid oriented 105W;	at 60N
		Pole at (58.5,92.5). (NGM "Big C-Grid")

	103	3640-point (65x56) N. Hemisphere polar	91.452 km
		stereographic grid oriented 105W; Pole	at 60N
		at (25.5,84.5) (used by ARL)

	104	16170-point (147x110) N. Hemisphere	90.75464 km
		polar stereographic grid oriented 105W;	at 60N
		pole at (75.5,109.5). (NGM Super C grid)

	105	6889-point (83x83) N. Hemisphere polar	90.75464 km
		stereographic grid oriented 105W;	at 60N
		pole at  (40.5,88.5).  (U.S. area
		subset of NGM Super C grid, used 
		by ETA model)

	106	19305 point (165x117) N. Hemisphere	45.37732 km
		stereographic grid oriented 105W;	at 60N
		pole at (80,176)
		Hi res. ETA (2 x resolution of Super C)

	107	11040 point (120x92) N. Hemisphere	45.37732 km
		stereographic grid oriented 105W;	at 60N
		pole at (46,167)
		subset of Hi res. ETA; for ETA & MAPS/RUC

	126	Global Gaussian T126 grid.  See GRIB 
		specifications below

	201-nnn	AWIPS grids. See specifications below.

	255	(non-standard grid - defined in the GDS)

NOTE ON NMC STORAGE GRIDS:

On the polar stereographic grids, the vector wind is resolved into u and v components with respect to the grid coordinates, i.e., u represents motion in the direction of increasing x (i) coordinate, v in the direction of increasing y (j).  On the latitude-longitude grids, u and v are true eastward and northward components, respectively.  However, take note of Table 7, below, which allows for the specification of other possibilities.

INTERNATIONAL EXCHANGE AND FAMILY OF SERVICES (FOS) GRIDS

VALUE  RESOLUTION      AREA              GRID          GRID

(degrees)     COVERAGE            SHAPE        POINTS

lon x lat     (degrees)        cols  rows

21    5.0 x 2.5   0-180E, 0-90N         37   36 + pole  1333

22    5.0 x 2.5   180W-0, 0-90N         37   36 + pole  1333

23    5.0 x 2.5   0-180E, 90S-0  pole + 37   36         1333

24    5.0 x 2.5   180W-0, 90S-0  pole + 37   36         1333

25    5.0 x 5.0   0-355E, 0-90N         72   18 + pole  1297

26    5.0 x 5.0   0-355E, 90S-0  pole + 72   18         1297

50    2.5 x 1.25             (see note iv)               964

61    2.0 x 2.0   0-180E, 0-90N         91   45 + pole  4096

62    2.0 x 2.0   180W-0, 0-90N         91   45 + pole  4096

63    2.0 x 2.0   0-180E, 90S-0  pole + 91   45         4096

64    2.0 x 2.0   180W-0, 90S-0  pole + 91   45         4096

255         (non-standard grid - defined in the GDS)

NOTES ON INTERNATIONAL EXCHANGE/FOS GRIDS:

(i)    The grid points are laid out in a linear array such that the longitude index (the columns) is the most rapidly varying.  For the northern hemisphere grids the first point in the record is at the intersection of the western-most meridian and southern-most circle of latitude; the last point is the single polar value (see note iii, below).  For the southern hemisphere grids the first point in the record is the single polar value (see note iii, below); the last point is at the intersection of the eastern-most meridian and northern-most circle of latitude.  For those familiar with FORTRAN subscripting conventions, longitude is the first subscript, latitude the second.

(ii)   In grids 21 through 26, and 61 through 64, the values on the shared boundaries are included in each area.

(iii)  The datum for the pole point is given only once in each grid.  The user must expand, if desired, the single pole point value to all the pole "points" at the pole row of a latitude-longitude grid.  Scalar quantity values are the same for all pole points on a the grid.  Wind components at the poles are given by the formulae:

u = -speed * sin(dd)   &   v = -speed * cos(dd)

where dd is the direction of the wind as reported according to the

specification of wind direction at the poles (refer to WMO Manual

on Codes <1>, code table 878).

The WMO convention can be given this operational definition:  At the North Pole, face into the wind and report the value of the west longitude meridian along which the wind is coming at you; at the South Pole do likewise but report the east longitude meridian value. This is equivalent to placing the origin of a right-handed Cartesian coordinate system on the North Pole with the y-axis pointing to the prime (0 degree) meridian and the x-axis pointing to the 90 degrees west meridian, and then resolving any vector wind at the pole point into components along those axes.  At the South Pole the coordinate axes are oriented such that the y-axis points toward 180 degrees west.  Those components are the u- and v-values given as the single pair of pole point winds in the GRIB format.

In terms of a longitude/latitude grid these are the wind components for the pole point at the 180 degree meridian.  For example, on a 2.5x2.5 degree northern hemisphere grid (145x37 points), with the abscissa along the equator and the ordinate along the prime meridian, the transmitted north pole wind components are those that belong at the gridpoint (73,37).  The wind components at the other grid points along the pole row may be obtained through suitable rotation of the coordinate system.  All the components at the pole row are, of course, simply representations of the same vector wind viewed from differing (rotated) coordinate systems.  In the southern hemisphere the analogous situation holds; the single set of transmitted pole point wind components belong at the gridpoint (73,1).

(iv)   Grid 50 is a set of points over the contiguous United States and environs on a grid extending from 20N (row No. 1) to 60N (row  No. 33) in 1.25 degree intervals.  The grid increases in longitudinal extent from south to north in the following manner:

ROWS   NO. POINTS   LONGITUDINAL EXTENT

1-4      22          122.5W  - 70.0W

5-8      24          125.0W  - 67.5W

9-12     26          127.5W  - 65.0W

13-16     28          130.0W  - 62.5W

17-20     30          132.5W  - 60.0W

21-24     32          135.0W  - 57.5W

25-28     34          137.5W  - 55.0W

29-33     36          140.0W  - 52.5W

WAFC/ICAO/INTERNATIONAL EXCHANGE/FOS GRIDS

(Grids 37 - 44)

µ §

Global Coverage of Grids
Octants of the Globe

In the figure the boxes indicate the location of the octants of the globe, the numbers are the corresponding grid identification numbers (PDS Octet 7), and the letters are the grid identification used in the WMO heading (see Appendix A).

The left and right meridional columns of each octant/grid are shared with the neighbors.

The basic grid point separation is 1.25x1.25 deg. on a latitude / longitude array, but the grid is "thinned" by reducing the number of points in each row as one goes northward (or southward) away from the equator.  The latitudinal increment is always 1.25 deg.; this results in 73 rows where the pole is included as a "row", not a single gridpoint.

The longitudinal spacing at the equator is also 1.25 deg.; there will be 73 gridpoints there in each octant.

The number of points on each latitudinal row, other than the equator, is given by this formula (using FORTRAN notation):

NPOINTS = IFIX(2.0 + (90.0/1.25) * COS(LATITUDE))

Thus at the pole there will be two gridpoints, one each at the meridians that delineate the edges of the octant.  The formula was worked out so that there will be (approximately) equal geographic separation between the grid points uniformly across the globe.

Because of variations in precision and roundoff error in different computers, the value of NPOINTS may vary by 1 at "critical" latitudes when calculated on various hardware platforms.  Here is a table of the exact values of NPOINTS as a function of latitude as used in the internationally exchanged grids.  These numbers will, of course, be found in the Grid Description Section of each GRIB bulletin.

Latitude Range	NPOINTS
inclusive
(north or south)

0.00 -  8.75	73
	10.00 - 12.50	72
	13.75 - 16.25	71
	17.50 - 18.75	70
	20.00 - 21.25	69
	22.50	68
	23.75 - 25.00	65
	30.00	64
	31.25	63
	32.50	62
	33.75	61
	35.00 - 36.25	60
	37.50	59
	38.75	58
	40.00	57
	41.25	56
	42.50	55
	43.75	54
	45.00	52
	46.25	51
	47.50	50
	48.75	49
	50.00	48
	51.25	47
	52.50	45
	53.75	44

Latitude Range	NPOINTS
inclusive
(north or south)

55.00	43
	56.25	42
	57.50	40
	58.75	39
	60.00	38
	61.25	36
	62.50	35
	63.75	33
	65.00	32
	66.25	30
	67.50	29
	68.75	28
	70.00	26
	71.25	25
	72.50	23
	73.75	22
	75.00	20
	76.25	19
	77.50	17
	78.75	16
	80.00	14
	81.25	12
	82.50	11
	83.75	 9
	85.00	 8
	86.25	 6
	87.50	 5
	88.75	 3
	90.00	 2

When all this is put together the result is that there are 3447 points of data actually transmitted in any individual GRIB bulletin containing one octant of the globe.

In the GRIB bulletins all of this information will be included in the Grid Description Section (GDS); the GDS must be included in order to describe the thinned or "quasi-regular" grid structure. See Section 2 and Table C for the general description of the GDS; what follows are the specific values of the variables in the GDS that describe these eight grids.

GDS Contents

Octets	Value or variable

	1-3	178 (length of GDS)
	4	0 (or 255, either indicating no PV)
	5	33 (pointer to start of PL list)
	6	0
	7-32	Grid description - see below
	33-178	number of points in each of 73 rows
		(2 octets per point)

Details of Octets 7-32 - Grid Description

Octets	Variable & Value

	7-8	Ni = 73
	9-10	Nj = 73

|  | GRID: | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11-13 | La1 = | 0 | 0 | 0 | 0 | 90S | 90S | 90S | 90S |
| 14-16 | Lo1 = | 330 | 60 | 150 | 240 | 330 | 60 | 150 | 240 |

17	Resolution & Component Flag = [10000000] (binary)

|  | GRID: | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18-20 | La2 = | 90N | 90N | 90N | 90N | 0 | 0 | 0 | 0 |
| 21-23 | Lo2 = | 60 | 150 | 240 | 330 | 60 | 150 | 240 | 330 |

24-25	Di = 1.25 deg
	26-27	Dj = 1.25 deg
	28	Scan Mode = [01000000] (binary)
	29-32	Set to 0 (unused)

Note that the scanning direction is from the bottom (south edge) to the top of the octant grids, regardless of the hemisphere.  Thus in the northern hemisphere the first 73 data points (in the BDS) will be the equatorial values and the last two will be the polar values.  The PL counts in the GDS octets 33-178 will, of course, indicate contain these numbers.

In the southern hemisphere, the first two data points will be the south pole values, and the last 73 points will be the equatorial values.  Octets 33-34 in the GDS will contain "2", octets 35-36 will contain a "3", and so on to octet 177-178 which will contain "73".

SELECTED NMC GRIDS DEFINED USING GRIB SPECIFICATIONS
(See Table C for definition of symbols)

VALUE    	   GRID DESCRIPTIONS

1	Tropical Strip
	(Mercator)

Ni =	73
	Nj =	23
	La1 =	 48.09S
	Lo1 =	  0.0E
	Res. & Comp. flag =	1 0 0 0 0 0 0 0
	La2 =	 48.09N
	Lo2 =	  0.0W
	Latin = 	 22.5
	Scanning Mode (Bits 1 2 3) = 0 1 0

Di  = Dj =	513.669 km

For reference here are the lat/lon values of the corners of the grid:
	(1,1)   = 	48.09S, 0.00E
	(1,23)  =	48.09N, 0.00E
	(73,23) = 	48.09N, 0.00W
	(73,1)  =	48.09S, 0.00W

The longitudinal grid spacing is 5.00 degrees.

87 	U.S. Area; used in MAPS/RUC 
	(60km at 40N)
 	(N. Hem. polar stereographic)

	Nx =	81
	Ny =	62
	La1 =	22.8756N
	Lo1 =	239.5089E = 120.4911W
	Res. & Comp. flag = 0 0 0 0 1 0 0 0
	Lov = 	255.000E = 105.000W
	Dx  = Dy =	68.153 km
	Projection Flag (Bit 1) = 0
	Scanning Mode (Bits 1 2 3) = 0 1 0

For reference here are the lat/lon values of the corners of the grid:
	(1,1)   = 	22.8756N, 120.4911W
	(1,62)  =	52,4887N, 136.5458W
	(81,62) = 	46.0172N,  60.8284W
	(81,1)  =	20.1284N,  81.2432W

The pole point is at 	(I,J) = 	(31.91,112.53)

98		   Global Gaussian Latitude/Longitude T62 Resolution

Ni =	192
	Nj =	94
	La1 =	 88.542N
	Lo1 =	  0.0E
	Res. & Comp. flag =	1 0 0 0 0 0 0 0
	La2 =	 88.542S
	Lo2 =	358.125E = 1.875W
	Di = 	1.875 degrees
	N  =	47 (number of lat. circles, pole
		    to equator)
	Scanning Mode = 00000000(NB:matrix style)

For reference here are the lat/lon values of the corners of the grid:
	(1,1)   = 	88.542N, 0.0E  (upper left)
	(1,190)  =	88.542S, 0.0E
	(384,190) = 	88.542S, 359.0625E
	(384,1)  =	88.542N, 359.0625E

-----------------------------------------------------------------

126		   Global Gaussian Latitude/Longitude T126 Resolution

Ni =	384
	Nj =	190
	La1 =	 89.277N
	Lo1 =	  0.0E
	Res. & Comp. flag =	1 0 0 0 0 0 0 0
	La2 =	 89.277S
	Lo2 =	359.0625E = 0.9375W
	Di = 	0.9375 degrees
	N  =	95 (# of lat circles pole
		    to equator)
	Scanning Mode = 00000000(NB:matrix style)

For reference here are the lat/lon values of the corners of the grid:
	(1,1)   = 	89.277N, 0.0E  (upper left)
	(1,190)  =	89.277S, 0.0E
	(384,190) = 	89.277S, 359.0625E
	(384,1)  =	89.277N, 359.0625E

AWIPS-90 STORAGE AND TRANSMISSION GRIDS

Note: The following grids are intended for use in the U.S. Weather Service's Advanced Weather Information Processing System for the 1990s (AWIPS-90).  Their definition is subject to change as the AWIPS-90 requirements are further refined.

VALUE    			   AWIPS GRID DESCRIPTIONS
		    (See Table C for definition of symbols)

201 	  Hemispheric 
 	(polar stereographic)

	Nx =	65
	Ny =	65
	La1 =	-20.826N = 20.826S
	Lo1 =	210.000E = 150.000W
	Res. & Comp. flag =	0 0 0 0 1 0 0 0
	Lov = 	255.000E = 105.000W
	Dx  = Dy =	381.000 km
	Projection Flag (Bit 1) = 0
	Scanning Mode (Bits 1 2 3) = 0 1 0

The pole point is at 	(I,J) = 	(33,33)

Map 201 is the same as NMC storage grid 27, except it is rotated to 105 deg. orientation.

-----------------------------------------------------------------

202	National - CONUS
	(polar stereographic)

	Nx =	65
	Ny =	43
	La1 =	  7.838N
	Lo1 =	218.972E = 141.028W
	Res. & Comp. flag =	0 0 0 0 1 0 0 0
	Lov = 	255.000E = 105.000W
	Dx  = Dy =	190.500 km
	Projection Flag (Bit 1) = 0
	Scanning Mode (Bits 1 2 3) = 0 1 0

For reference here are the lat/lon values of the corners of the grid:
	(1,1)   = 	 7.838N, 141.028W
	(1,43)  =	35.616N, 168.577E
	(65,43) = 	35.617N,  18.576W
	(65,1)  =	 7.838N,  68.973W

The pole point is at 	(I,J) = 	(33,45)

203	National - Alaska
	(polar stereographic)

	Nx =	45
	Ny =	39
	La1 =	 19.132N
	Lo1 =	174.163E = 185.837W
	Res. & Comp. flag =	0 0 0 0 1 0 0 0
	Lov = 	210.000E = 150.000W
	Dx  = Dy =	190.500 km
	Projection Flag (Bit 1) = 0
	Scanning Mode (Bits 1 2 3) = 0 1 0

For reference here are the lat/lon values of the corners of the grid:
	(1,1)   = 	19.132N, 174.163E
	(1,39)  =	44.646N, 115.601E
	(45,39) = 	57.634N,  53.660W
	(45,1)  =	24.361N, 123.434W

The pole point is at 	(I,J) = 	(27,37)

-----------------------------------------------------------------

204	National - Hawaii
	(Mercator)

Ni =	93
	Nj =	68
	La1 =	 25.000S
	Lo1 =	110.000E
	Res. & Comp. flag =	1 0 0 0 0 0 0 0
	La2 =	 60.644N
	Lo2 =	109.129W
	Latin = 	 20.000
	Scanning Mode (Bits 1 2 3) = 0 1 0

Di  = Dj =	160.000 km

For reference here are the lat/lon values of the corners of the grid:
	(1,1)   = 	25.000S, 110.000E
	(1,68)  =	60.644N, 110.000E
	(93,68) = 	60.644N, 109.129W
	(93,1)  =	25.000S, 109.129W

The longitudinal grid spacing is 1.531 degrees.

205	National - Puerto Rico
	(polar stereographic)

Nx =	 45
	Ny =	 39
	La1 =	  0.616N
	Lo1 =	275.096E = 84.904W
	Res. & Comp. flag =	0 0 0 0 1 0 0 0
	Lov = 	300.000E = 60.000W
	Dx  = Dy =	190.500 km
	Projection Flag (Bit 1) = 0
	Scanning Mode (Bits 1 2 3) = 0 1 0

For reference here are the lat/lon values of the corners of the grid:

	(1,1)   = 	 0.616N,  84.904W
	(1,39)  =	36.257N, 115.304W
	(45,39) = 	45.620N,  15.000W
	(45,1)  =	 3.389N,  42.181W

The pole point is at 	(I,J) = 	(27,57)

-----------------------------------------------------------------

206	Regional - Central US MARD 
	(Lambert Conformal)

Nx =	51
	Ny =	41
	La1 =	 22.289N
	Lo1 =	242.009E = 117.991W
	Res. & Comp. flag =	0 0 0 0 1 0 0 0
	Lov = 	265.000E = 95.000W
	Dx = Dy =	 81.2705 km
	Projection Flag = 0  (not bipolar)
	Scanning Mode (Bits 1 2 3) = 0 1 0

Latin 1 = 	 25.000N
	Latin 2 =    25.000N   (tangent cone)

For reference here are the lat/lon values of the corners of the grid:
	(1,1)   = 	22.289N, 117.991W
	(1,41)  =	50.081N, 124.898W
	(51,41) = 	50.072N,  73.182W
	(51,1)  =	23.142N,  78.275W

The Dx, Dy grid increment (at 25 deg north) was selected so that the grid spacing would be exactly 80.000 km at 35 deg north; the intersection of 35N & 95W falls on point (30,16).

207	Regional - Alaska
	(polar stereographic)

Nx =	 49
	Ny =	 35
	La1 =	 42.085N
	Lo1 =	184.359E = 175.641W
	Res. & Comp. flag =	0 0 0 0 1 0 0 0
	Lov = 	210.000E = 150.000W
	Dx  = Dy =	 95.250 km
	Projection Flag (Bit 1) = 0
	Scanning Mode (Bits 1 2 3) = 0 1 0

For reference here are the lat/lon values of the corners of the grid:
	(1,1)   = 	42.085N, 175.641W
	(1,35)  =	63.976N, 153.689E
	(49,35) = 	63.976N,  93.689W
	(49,1)  =	42.085N, 124.359W

The pole point is at 	(I,J) = 	(25,51)

-----------------------------------------------------------------

208	Regional - Hawaii
	(Mercator)

Ni =	29
	Nj =	27
	La1 =	  9.343N
	Lo1 =	192.685E = 167.315W
	Res. & Comp. flag =	1 0 0 0 0 0 0 0
	La2 =	 28.092N
	Lo2 =	145.878W
	Latin = 	 20.000
	Scanning Mode (Bits 1 2 3) = 0 1 0
	Di  = Dj =	 80.000 km

For reference here are the lat/lon values of the corners of the grid:
	(1,1)   = 	 9.343N, 167.315W
	(1,27)  =	28.092N, 167.315W
	(29,27) = 	28.092N, 145.878W
	(29,1)  =	 9.343N, 145.878W

The longitudinal grid spacing is 0.766 degrees.  The grid is positioned such that the odd-numbered rows and columns coincide with the National grid, No. 204;  the lower left corner of the regional grid is located at National (204) grid-point (55,24) and the upper right corner is located at (69,37).

209	Regional - Central US MARD - Double Res.
	(Lambert Conformal)

Nx =	101
	Ny =	81
	La1 =	 22.289N
	Lo1 =	242.00962E = 117.991W
	Res. & Comp. flag =	0 0 0 0 1 0 0 0
	Lov = 	265.000E =  95.000W
	Dx = Dy =	 40.63525 km
	Projection Flag = 	0 (not bipolar)
	Scanning Mode (Bits 1 2 3) = 0 1 0

Latin 1 = 	 25.000N
	Latin 2 =    25.000N   (tangent cone)

For reference here are the lat/lon values of the corners of the grid:
	(1,1)    = 	22.289N, 117.991W
	(1,81)   =	50.081N, 124.898W
	(101,81) = 	50.072N,  73.182W
	(101,1)  =	23.142N,  78.275W

The Dx, Dy grid increment (at 25 deg north) was selected so that the grid spacing would be exactly 40.000 km at 35 deg north; the intersection of 35N & 95W falls on point (59,31).

-----------------------------------------------------------------

210	Regional - Puerto Rico
	(Mercator)

Ni =	25
	Nj =	25
	La1 =	  9.000N
	Lo1 =	283.000E = 77.000W
	Res. & Comp. flag =	1 0 0 0 0 0 0 0
	La2 =	 26.422N
	Lo2 =	 58.625W
	Latin = 	 20.000
	Di  = Dj =	 80.000 km
	Scanning Mode (Bits 1 2 3) = 0 1 0

For reference here are the lat/lon values of the corners of the grid:
	(1,1)   = 	 9.000N, 77.000W
	(1,25)  =	26.422N, 77.000W
	(25,25) = 	26.422N, 58.625W
	(25,1)  =	 9.000N, 58.626W

The longitudinal grid spacing is 0.766 degrees

211	Regional - CONUS
	(Lambert Conformal)

Nx =	93
	Ny =	65
	La1 =	 12.190N
	Lo1 =	226.541E = 133.459W
	Res. & Comp. flag =	0 0 0 0 1 0 0 0
	Lov = 	265.000E = 95.000W
	Dx = Dy =	 81.2705 km
	Projection Flag = 0  (not bipolar)
	Scanning Mode (Bits 1 2 3) = 0 1 0

Latin 1 = 	 25.000N
	Latin 2 =    25.000N   (tangent cone)

For reference here are the lat/lon values of the corners of the grid:
	(1,1)    = 	12.190N, 133.459W
	(1,65)   =	54.536N, 152.856W
	(93,65)  = 	57.290N,  49.385W
	(93,1)   =	14.335N,  65.091W

The Dx, Dy grid increment (at 25 deg north) was selected so that the grid spacing would be exactly 80.000 km at 35 deg north; the intersection of 35N & 95W falls on point (53,25).

-----------------------------------------------------------------

212				   Regional  - CONUS - double resolution

(Lambert Conformal)

Nx =	185
	Ny =	129
	La1 =	 12.190N
	Lo1 =	226.514E = 133.459W
	Res. & Comp. flag =	0 0 0 0 1 0 0 0
	Lov = 	265.000E =  95.000W
	Dx = Dy =	 40.63525 km
	Projection Flag = 0  (not bipolar)
	Scanning Mode (Bits 1 2 3) = 0 1 0

Latin 1 = 	 25.000N
	Latin 2 =    25.000N   (tangent cone)

For reference here are the lat/lon values of the corners of the grid:
	(1,1)     = 	12.190N, 133.459W
	(1,129)   =	   54.536N, 122.856W
	(185,129) = 	57.290N,  49.385W
	(185,1)   =		14.335N,  65.091W

The Dx, Dy grid increment (at 25 deg north) was selected so that the grid spacing would be exactly 40.000 km at 35 deg north; the intersection of 35N & 95W falls on point (105,49).

213	National - CONUS - Double Resolution
	(polar stereographic)

	Nx =	129
	Ny =	85
	La1 =	  7.838N
	Lo1 =	218.972E = 141.028W
	Res. & Comp. flag =	0 0 0 0 1 0 0 0
	Lov = 	255.000E = 105.000W
	Dx  = Dy =	 95.250 km
	Projection Flag (Bit 1) = 0
	Scanning Mode (Bits 1 2 3) = 0 1 0

For reference here are the lat/lon values of the corners of the grid:
	(1,1)    =	 7.838N, 141.028W
	(1,85)   =	35.617N, 168.577E
	(129,85) =	35.617N,  18.577W
	(129,1)  =	 7.838N,  68.973W

The pole point is at 	(I,J) = 	(65,89)

-----------------------------------------------------------------

214	Regional - Alaska - Double Resolution

(polar stereographic)

Nx =	 97
	Ny =	 69
	La1 =	 42.085N
	Lo1 =	184.359E = 175.641W
	Res. & Comp. flag =	0 0 0 0 1 0 0 0
	Lov = 	210.000E = 150.000W
	Dx  = Dy =	 47.625 km
	Projection Flag(Bit 1) = 0
	Scanning Mode (Bits 1 2 3) = 0 1 0

For reference here are the lat/lon values of the corners of the grid:
	(1,1) = 	42.085N, 175.641W

(1,69) =	63.975N, 153.690E
	(97,69) = 	63.975N,  93.689W
	(97,1) =	42.085N, 124.358W

The pole point is at 	(I,J) = 	(49,101)

TABLE 1.  FLAG FOR GDS OR BMS
(PDS Octet 8)

The bit flag indicates the omission or inclusion of the Grid Description and/or Bit Map Sections.

BIT	VALUE	MEANING

1	  0	GDS Omitted
		  1	GDS Included

	2	  0	BMS Omitted
		  1	BMS Included

	3-8	  0	reserved

TABLE 2.  PARAMETERS & UNITS
Version 1
(PDS Octet 9)

VALUE               PARAMETER                 UNITS

000     Reserved

001     Pressure                               Pa

002     Pressure reduced to MSL                Pa

003     Pressure tendency                      Pa/s

004

005

006     Geopotential                          m2/s2

007     Geopotential height                    gpm

008     Geometric height                       m

009

010

011     Temperature                            deg. K

012     Virtual temperature                    deg. K

013     Potential temperature                  deg. K

014     Pseudo-adiabatic potential temperature deg. K

015     Maximum temperature                    deg. K

016     Minimum temperature                    deg. K

017     Dew point temperature                  deg. K

018     Dew point depression (or deficit)      deg. K

019     Lapse rate                           deg. K/m

020

021     Radar Spectra (1)					-

022     Radar Spectra (2)					-

023     Radar Spectra (3)					-

024

025     Temperature anomaly                    deg. K

026	    Pressure anomaly				     Pa

027	    Geopotential height anomaly		     gpm

028     Wave Spectra (1)				      -

029     Wave Spectra (2)				      -

030     Wave Spectra (3)				      -

031     Wind direction                     deg. true

032     Wind speed                             m/s

033     u-component of wind                    m/s

034     v-component of wind                    m/s

035     Stream function                        m2/s

036     Velocity potential                     m2/s

037     Montgomery stream function             m2/s2

038     Sigma coord. vertical velocity          /s

039     Pressure Vertical velocity            Pa/s

040     Geometric Vertical velocity            m/s

041     Absolute vorticity                     /s

042     Absolute divergence                    /s

043     Relative vorticity                     /s

044     Relative divergence                    /s

045     Vertical u-component shear             /s

046     Vertical v-component shear             /s

047     Direction of current                deg. true

048     Speed of current                       m/s

049     u-component of current                 m/s

050     v-component of current                 m/s

051     Specific humidity                      kg/kg

052     Relative humidity                      %

053     Humidity mixing ratio                  kg/kg

054     Precipitable water                     kg/m2

055     Vapor pressure                         Pa

056     Saturation deficit                     Pa

057     Evaporation                            kg/m2

058

059	    Precipitation rate                   kg/m2/s

060     Thunderstorm probability			   %

061     Total precipitation                   kg/m2

062     Large scale precipitation             kg/m2

063     Convective precipitation              kg/m2

064     Snowfall rate water equivalent        kg/m2s

065     Water equiv. of accum. snow depth     kg/m2

066     Snow depth                             m

067     Mixed layer depth                      m

068     Transient thermocline depth            m

069     Main thermocline depth                 m

070     Main thermocline anomaly               m

071     Total cloud cover                      %

072     Convective cloud cover                 %

073     Low cloud cover                        %

074     Medium cloud cover                     %

075     High cloud cover                       %

076     Cloud water                            kg/m2

077     Condensation pressure of parcel        Pa
		    lifted from indicated surface

078

079

080     Water Temperature				  deg K

081     Land-sea mask (1=land; 0=sea)         1/0

082     Deviation of sea level from mean       m

083     Surface roughness                      m

084     Albedo                                 %

085     Soil temperature                      deg. K

086     Soil moisture content                 kg/m2

087     Vegetation                             %

088     Salinity                              kg/kg

089     Density                               kg/m3

090

091     Ice concentration (ice=1; no ice=0)    1/0

092     Ice thickness                          m

093     Direction of ice drift              deg. true

094     Speed of ice drift                     m/s

095     u-component of ice drift               m/s

096     v-component of ice drift               m/s

097     Ice growth rate                        m/s

098     Ice divergence                         /s

099

100     Significant height of combined wind    m

waves and swell

101     Direction of wind waves             deg. true

102     Significant height of wind waves       m

103     Mean period of wind waves              s

104     Direction of swell waves            deg. true

105     Significant height of swell waves      m

106     Mean period of swell waves             s

107     Primary wave direction              deg. true

108     Primary wave mean period               s

109     Secondary wave direction            deg. true

110     Secondary wave mean period             s

111     Net short-wave radiation (surface)      W/m2

112     Net long wave radiation (surface)       W/m2

113     Net short-wave radiation (top of atmos.)W/m2

114     Net long wave radiation (top of atmos.) W/m2

115     Long wave radiation                    W/m2

116     Short wave radiation                   W/m2

117     Global radiation                       W/m2

118

119

120

121     Latent heat flux                       W/m2

122     Sensible heat flux                     W/m2

123     Boundary layer dissipation             W/m2

124

125

126
      127	    Image data

(128 - 254  Reserved for use by originating center)

NWS/NMC usage as follows...

128	Mean Sea Level Pressure (MSLSA)	Pa
		(Standard Atmosphere Reduction)

129	Mean Sea Level Pressure (MSLMA)	Pa
		(MAPS System Reduction)

130	Mean Sea Level Pressure  (MSLET)	Pa
		(ETA Model Reduction)
	131	Surface lifted index	Deg. K
	132	Best (4 layer) lifted index	Deg. K
	133	K index	Deg. K
	134	Sweat index	Deg. K
	135	Horizontal moisture divergence	kg/kg/s
	136	Vertical speed shear	/s
	137	Visibility	m

	150	Covariance between meridional	m2/s2
		and zonal components of the wind.
		Defined as [uv]-[u][v], where
		"[]" indicates the mean over the
		indicated time span.

	151	Covariance between temperature	K*m/s
		and zonal component of the wind.
		Defined as [uT]-[u][T], where
		"[]" indicates the mean over the
		indicated time span.

	152	Covariance between temperature	K*m/s
		and meridional component of the
		wind.  Defined as [vT]-[v][T], 
		where "[]" indicates the mean 
		over the indicated time span.

	157	Convective Available Potential Energy	J/kg
	158	Turbulent Kinetic Energy	J/kg
	
	176	latitude (-90 to +90) (NLAT)	deg
	177	east longitude (0-360)(ELON)	deg

	201	Ice-free water surface	%
	204	downward short wave rad. flux (DSWRF)	W/m**2
	205	downward long wave rad. flux (DLWRF)	W/m**2
	207	Moisture availability	%
	208	Exchange coefficient                (kg/m3)(m/s)
	209	No. of mixed layers next to surface	integer
	211	upward short wave rad. flux (USWRF)	W/m**2
	212	upward long wave rad. flux (ULWRF)	W/m**2
	213	Amount of non-convective cloud	%
	216	Temperature tendency by all radiation	Deg.K/s
	218	precip.index(0.0-1.00)(see note)(PREIX) fraction
	220	Natural log of surface pressure	ln(kPa)
	222	5-wave geopotential height	gpm

	255	Missing

Notes:

1)	By convention, downward fluxes of radiation or other quantities are assigned negative values; upward fluxes of radiation or other quantities are assigned positive values.

2)	The u and v components of vector quantities are defined with reference to GDS Octet 17 and Table 7.

3)	provision is made for three types of spectra:

1)	Direction and Frequency
2)	Direction and radial number
3)	Radial number and radial number

4) Precipitation index (#218) defined as the fraction of satellite observed pixels with temperatures <235K over 1.0x1.0 box, centered at the gridpoint.

TABLE 3.  TYPE AND VALUE OF LEVEL
(PDS Octets 10, 11, & 12)

Octet Number 10            Number 11        Number 12

VALUE	   MEANING                      CONTENTS

|---------------------------+-----------------+----------------|

| 0 - 99  special codes,    |        0        |       0        |

|         see Table 3a      |                 |                |

|---------------------------+-----------------+----------------|

| 100     isobaric level    |  pressure in hectoPascals (hPa)  |

|                           |            (2 octets)            |

|---------------------------+-----------------+----------------|

| 101     layer between two |  pressure of    |  pressure of   |

|         isobaric levels   |  top (kPa)      |  bottom (kPa)  |

|---------------------------+-----------------+----------------|

| 102     mean sea level    |        0        |       0        |

|---------------------------+-----------------+----------------|

| 103     fixed height level|   height above mean sea level    |

|                           |        (MSL) in meters           |

|---------------------------+-----------------+----------------|

| 104     layer between two |  height of top  |  height of     |

|           height levels   |     (hm)        |  bottom (hm)   |

|            above msl      |                 |                |

|---------------------------+-----------------+----------------|

| 105     fixed height above|         height in meters         |

|           ground          |            (2 octets)            |

|---------------------------+-----------------+----------------|

| 106     layer between two |  height of top  |  height of     |

|           height levels   |      (hm)       |  bottom (hm)   |

|           above ground    |                 |                |

|---------------------------+-----------------+----------------|

| 107     sigma level       |     sigma value in 1/10000       |

|                           |            (2 octets)            |

|---------------------------+-----------------+----------------|

| 108     layer between two |  sigma value at |  sigma value   |

|           sigma levels    |       top       |  at bottom     |

|                           |     in 1/100    |   in 1/100     |

|---------------------------+-----------------+----------------|

| 109     Hybrid level      |           level number           |

|                           |            (2 octets)            |

|---------------------------+-----------------+----------------|

| 110     layer between two | level number of | level number   |

|           hybrid levels   |       top       |  of bottom     |

|---------------------------+-----------------+----------------|

| 111     depth below       |            centimeters           |

|         land surface      |             (2 octets)           |

|---------------------------+-----------------+----------------|

| 112     layer between two |  depth of upper | depth of lower |

|         depths below      |    surface (cm) |   surface (cm) |

|         land surface      |                 |                |

|---------------------------+-----------------+----------------|

|---------------------------+-----------------+----------------|

| 113     isentropic        |     Potential Temp. degrees K    |

|         (theta) level     |             (2 octets)           |

|---------------------------+-----------------+----------------|

| 114     layer between two |    475K minus   |  475K minus    |

|         isentropic        |   theta of top  |theta of bottom |

|         levels            |    in Deg. K    |   in deg. K    |

|---------------------------+-----------------+----------------|

| 121     layer between two |  1100 hPa minus | 1100 hPa minus |

|         isobaric surfaces |  pressure of    | pressure of    |

|         (high precision)  |  top, in hPa    | bottom, in hPa |

|---------------------------+-----------------+----------------|

| 128     layer between two |  1.1 minus sigma| 1.1 minus sigma|

|         sigma levels      |  of top, in     | of bottom, in  |

|         (high precision)  |  1/1000 of sigma| 1/1000 of sigma|

|---------------------------+-----------------+----------------|

| 141     layer between two |  pressure of    | 1100hPa minus  |

|         isobaric surfaces |  top, in kPa    | pressure of    |

|         (mixed precision) |                 | bottom, in hPa |

|---------------------------+-----------------+----------------|

| 160     depth below       |               meters             |

|         sea level         |             (2 octets)           |

|---------------------------+-----------------+----------------|

Note: The numbering allows for additions within this framework:

100-119	normal precision
120-139	high precision
140-159	mixed precision

TABLE 3a.   SPECIAL LEVELS
(PDS Octet 10)

VALUE               LEVEL

01                surface (of the Earth, which

includes sea surface)

02                cloud base level

03                cloud top level

04                0 deg (C) isotherm level

05                adiabatic condensation level
			  (lifted from boundary layer)

06                maximum wind speed level

07                tropopause level

08-99             reserved

TABLE 4.  FORECAST TIME UNIT
(PDS Octet 18)

VALUE			TIME UNIT

0			minute

1			hour

2			day

3			month

4			year

5			decade

6			normal (30 years)

7			century

8-253		reserved

254			second

TABLE 5. 
TIME RANGE INDICATOR
(PDS Octet 21)

VALUE			MEANING

0	Forecast product valid 
		at reference time + P1 (P1>0), 
		or
		Uninitialized analysis product 
		for reference time (P1=0).
		or
		Image product 
		for reference time (P1=0)

	1	Initialized analysis product
		for reference time (P1=0).

	2	Product with a valid time ranging  				between reference time + P1 and
		reference time + P2

	3	Average
		(reference time + P1 to
		reference time + P2)

	4	Accumulation 
		(reference time + P1 to
		reference time + P2)
		product considered valid at 
		reference time + P2

	5	Difference
		(reference time + P2 minus
		reference time + P1)
		product considered valid at 
		reference time + P2

	6-9	reserved

	10	P1 occupies octets 19 and 20;
		product valid at reference time + P1

11-112	reserved

113	Average of N forecasts (or initialized
		analyses); each product has forecast
		period of P1 (P1=0 for initialized
		analyses); products have reference
		times at intervals of P2, beginning at
		the given reference time.

	114	Accumulation of N forecasts (or
		initialized analyses); each product
		has forecast period of P1 (P1=0 for
		initialized	analyses); products have
		reference times at intervals of P2,
		beginning at the given reference time.

	115	Average of N forecasts, all with the
		same reference time; the first has a
		forecast period of P1, the remaining
		forecasts follow at intervals of P2.

116	Accumulation of N forecasts, all with
		the same reference time; the first has
		a	forecast period of P1, the remaining
		follow at intervals of P2.

117	Average of N forecasts, the first has a period of P1, the subsequent ones have forecast periods reduced from the previous one by an interval of P2; the reference time for the first is given in octets 13-17, the subsequent ones have reference times increased from the previous one by an interval of P2.  Thus all the forecasts have the same valid time, given by the initial reference time + P1.

118	Temporal variance, or covariance, of N initialized analyses; each product has forecast period P1=0; products have reference times at intervals of P2, beginning at the given reference time.

119 -122	reserved

	123	Average of N uninitialized	analyses,
		starting at the reference time,
		at intervals of P2.

124	Accumulation of N uninitialized
		analyses, starting at the reference
		time, at intervals of P2.

125-254	reserved

NOTES:

1)	For analysis products, or the first of a series of analysis products, the reference time (octets 13 to 17) indicates the valid time.

2)	For forecast products, or the first of a series of forecast products, the reference time indicates the valid time of the analysis upon which the (first) forecast is based.

3)	Initialized analysis products are allocated numbers distinct from those allocated to uninitialized analysis products.

4)	A value of 10 allows the period of a forecast to be extended over two octets; this is to assist with extended range forecasts.

5)	Where products or a series of products are averaged or accumulated, the number involved is to be represented in octets 22-23 of Section 1, while any number missing is to be represented in octet 24.

6)	Forecasts of the accumulation or difference of some quantity (e.g. quantitative precipitation forecasts), indicated by values of 4 or 5 in octet 21, have a product valid time given by the reference time + P2; the period of accumulation, or difference, can be calculated as P2 - P1.

A few examples may help to clarify the use of Table 5:

For analysis products P1 is zero and the time range indicator is also zero; for initialized products (sometimes called "zero hour forecasts") P1 is zero, but octet 21 is set to 1.

For forecasts, typically, P1 contains the number of hours of the forecast (the unit indicator given in octet 18 would be 1) and octet 21 contains a zero.

Value 115 would be used, typically, for multiple day mean forecasts, all derived from the same initial conditions.

Value 117 would be used, typically, for Monte Carlo type calculations: many forecasts valid at the same time from different initial (reference) times.

Averages, accumulations, and differences get a somewhat specialized treatment.  If octet 21 (Table 5) has a value between 2 and 5 (inclusive) then the reference time + P1 is the initial date/time and the reference time + P2 is the final date/time of the period over which averaging or accumulation takes place.  If, however, octet 21 has a value of 113, 114, 115, 116, 117, 118, 123, or 124 then P2 specifies the time interval between each of the fields (or the forecast initial times) that have been averaged or accumulated.  These latter values of octet 21 require the quantities averaged to be equally separated in time; the former values, 3 and 4 in particular, allow for irregular or unspecified intervals of time between the fields that are averaged or accumulated.

section 2: GRID DESCRIPTION SECTION (GDS)

The purpose of the (optional) GDS is to provide a grid description for grids not defined by number in Table 3.

Octet no.					GDS Content

1 - 3        Length in octets of the Grid Description Section

4		   NV, the number of vertical coordinate parameters

5	PV, the location (octet number) of the list of
	vertical coordinate parameters, if present
	or
	PL, the location (octet number) of the list of
	numbers of points in each row (when no vertical
	parameters are present), if present
	or
	255 (all bits set to 1) if neither are present

6		   Data representation type (See Table 6)

7 - 32      Grid description, according to data
			   representation type, except Lambert or Mercator.

or

7 - 42	   Grid description for Lambert or Mercator grid

PV	List of vertical coordinate parameters
	(length = NV x 4 octets);
	if present, then PL = 4 x NV + PV

PL	List of numbers of points in each row, used for
	quasi-regular grids
	(length = NROWS x 2 octets, where NROWS is the
	total number of rows defined within the grid
	description)

Note: 	NV and PV relate to "advanced" features of GRIB not, at present, in use in the National Weather Service.  See the WMO Manual on Codes<1> for the descriptions of those features.

PL is used for "quasi-regular" or "thinned" grids; e.g., a lat/lon grid where the number of points in each row is reduced as one moves poleward from the equator.  The reduction usually follows some mathematical formula involving the cosine of the latitude, to generate an (approximately) equally spaced grid array.  The association of the numbers in octet PL (and following) with the particular row follows the scanning mode specification in Table 8.

TABLES FOR THE GDS

TABLE 6.  DATA REPRESENTATION TYPE
(GDS Octet 6)

VALUE	MEANING

0	Latitude/Longitude Grid
		also called Equidistant Cylindrical
		or Plate Carree projection grid
	1	Mercator Projection Grid
	2	Gnomonic Projection Grid
	3	Lambert Conformal, secant or
		tangent, conical or bipolar 
		(normal or oblique) Projection Grid
	4	Gaussian Latitude/Longitude Grid
	5	Polar Stereographic Projection Grid

	6 - 12	(reserved - see Manual on Codes)

	13	Oblique Lambert conformal, secant or
		tangent, conical or bipolar,
		projection

	14 - 49	(reserved - see Manual on Codes)

	50	Spherical Harmonic Coefficients

	51 - 89	(reserved - see Manual on Codes)

	90	Space view perspective or
		orthographic grid

	91 - 254	(reserved - see Manual on Codes)

TABLE C. Sundry Grid Definitions

LATITUDE/LONGITUDE GRIDS
INCLUDING GAUSSIAN
(GDS Octets 7 - 32)

OCTET NO.	CONTENT & MEANING

 7 -  8	Ni - No. of points along a latitude circle

	9 - 10	Nj - No. of points along a longitude meridian

	11 - 13	La1 - latitude of first grid point
		units: millidegrees (degrees x 1000)
		values limited to range 0 - 90,000 
		bit 1 (leftmost) set to 1 for south latitude

	14 - 16	Lo1 - longitude of first grid point
		units: millidegrees (degrees x 1000)
		values limited to range 0 - 360,000 
		bit 1 (leftmost) set to 1 for west longitude

	17	Resolution and component flags (Table 7)
		
	18 - 20	La2 - Latitude of last grid point
		(same units, value range, and bit 1 as La1)

	21 - 23	Lo2 - Longitude of last grid point
		(same units, value range, and bit 1 as Lo1)

	24 - 25	Di - Longitudinal Direction Increment 
		(same units as Lo1)
		(if not given, all bits set = 1)

 	26 - 27	Regular Lat/Lon Grid:
			Dj - Latitudinal Direction Increment
			(same units as La1)
			(if not given, all bits set = 1)
			or
			Gaussian Grid:
			N - number of latitude circles between
			    a pole and the equator
			Mandatory if Gaussian Grid specified 

	 28	Scanning mode flags  (See Table 8)

	 29 - 32	Reserved (set to zero)

Note: The latitude and longitude of the first and last grid points should always be given, for regular grids.

POLAR STEREOGRAPHIC GRIDS
(GDS Octets 7 - 32)

OCTET NO.	CONTENT & MEANING

	 7 -  8	Nx - Number of points along x-axis
	 9 - 10	Ny - Number of points along y-axis
	11 - 13	La1 - Latitude of first grid point
	14 - 16	Lo1 - Longitude of first grid point
	     17	Resolution and component flags (see Table 7)
	18 - 20	Lov - The orientation of the grid; 
		i.e., the east longitude value of the
		meridian which is parallel to the 
		y-axis (or columns of the grid) along
		which latitude increases as the 
		y-coordinate increases. (Note: The
		orientation longitude may, or may not,
		appear within a particular grid.)
	21 - 23	Dx - the X-direction grid length 
			  (see Note 2)
	24 - 26	Dy - the Y-direction grid length 
			  (see note 2)
	     27	Projection center flag  (see note 5)
	     28	Scanning mode           (see Table 8)
	29 - 32	Set to 0 (reserved)

NOTES:

1.	Latitude and longitude are in millidegrees (thousandths)

2.	Grid lengths are in units of meters, at the 60 degree latitude circle nearest to the pole in the projection plane.

3.	Latitude values are limited to the range 0 - 90,000.  Bit 1 is set to 1 to indicate south latitude.

4.	Longitude values are limited to the range 0 - 360,000.  Bit one is set to 1 to indicate west longitude.

5.	Octet 27:
Bit 1 set to 0 if the North pole is on the projection plane.
Bit 1 set to 1 if the South pole is on the projection plane.

6.	The first and last grid points may not necessarily be the same as the first and last data points if the bit map section (BMS) is used.

7.	The resolution flag (bit 1 of Table 7) is not applicable.

LAMBERT CONFORMAL SECANT OR TANGENT CONE GRIDS
(GDS Octets 7 - 42)

OCTET NO.	CONTENT & MEANING

	 7 -  8	Nx - Number of points along x-axis
	 9 - 10	Ny - Number of points along y-axis
	11 - 13	La1 - Latitude of first grid point
	14 - 16	Lo1 - Longitude of first grid point
	     17	Resolution and component flags (see Table 7)
	18 - 20	Lov - The orientation of the grid; 
		i.e., the east longitude value of the
		meridian which is parallel to the 
		y-axis (or columns of the grid) along
		which latitude increases as the 
		y-coordinate increases. (Note: The
		orientation longitude may, or may not,
		appear within a particular grid.)
	21 - 23	Dx - the X-direction grid length 
			  (see note 2)
	24 - 26	Dy - the Y-direction grid length 
			  (see Note 2)
	     27	Projection center flag   (see note 5)
	     28	Scanning mode            (see Table 8)
	29 - 31	Latin 1 - The first latitude from the pole
		at which the secant cone cuts the spherical
		earth.       (See Note 8)
	32 - 34	Latin 2 - The second latitude from the pole
		at which the secant cone cuts the spherical
		earth.       (See Note 8)

35 - 37	Latitude of southern pole (millidegrees)
	38 - 40	Longitude of southern pole (millidegrees)
	41 - 42	Reserved (set to 0)

NOTES:

1.	Latitude and longitude are in millidegrees (thousandths)

2.	Grid lengths are in units of meters, at the intersection latitude circle nearest to the pole in the projection plane.

3.	Latitude values are limited to the range 0 - 90,000.  Bit 1 is set to 1 to indicate south latitude.

4.	Longitude values are limited to the range 0 - 360,000.  Bit one is set to 1 to indicate west longitude.

5.	Octet 27:
Bit 1 set to 0 if the North pole is on the projection plane.
Bit 1 set to 1 if the South pole is on the projection plane.
Bit 2 set to 0 if only one projection center used
Bit 2 set to 1 if projection is bipolar and symmetric

6.	The first and last grid points may not necessarily be the same as the first and last data points if the bit map section (BMS) is used.

7.	The resolution flag (bit 1 of Table 7) is not applicable.

8.	If Latin 1 = Latin 2 then the projection is on a tangent cone.

MERCATOR GRIDS
(GDS Octets 7 - 42)

OCTET NO.	CONTENT & MEANING

	 7 -  8	Ni - Number of points along a
			  latitude circle
	 9 - 10	Nj - Number of points along a
			  longitude meridian
	11 - 13	La1 - Latitude of first grid point
	14 - 16	Lo1 - Longitude of first grid point
	     17	Resolution and component flags (see Table 7)
	18 - 20	La2 - latitude of last grid point
	21 - 23	Lo2 - longitude of last grid point
	24 - 26	Latin - The latitude(s) at which the 
				Mercator projection cylinder
				intersects the earth.  
	     27	Reserved (set to 0)
	     28	Scanning mode (see Table 8)
	29 - 31	Di - the longitudinal direction increment 
			  (see Note 2)
	32 - 34	Dj - the latitudinal direction increment 
			  (see note 2)
	35 - 42	Reserved (set to 0)

NOTES:

1.	Latitude and longitude are in millidegrees (thousandths)

2.	Grid lengths are in units of meters, at the circle of latitude specified by Latin.

3.	Latitude values are limited to the range 0 - 90,000.  Bit 1 is set to 1 to indicate south latitude.

4.	Longitude values are limited to the range 0 - 360,000.  Bit one is set to 1 to indicate west longitude.

5.	The latitude and longitude of the last grid point should always be given.

6.	The first and last grid points may not necessarily be the same as the first and last data points if the bit map section (BMS) is used.

SPHERICAL HARMONIC COEFFICIENTS
(GDS Octets 7 - 32)

OCTET NO.	CONTENT & MEANING

	 7 -  8	J - Pentagonal Resolution Parameter

	 9 - 10	K - Pentagonal Resolution Parameter

	11 - 12	M - Pentagonal Resolution Parameter

	13	Representation Type (See Table 9)

	14	Coefficient Storage Mode (See Table 10)

	15 - 32	Set to zero (reserved)

TABLE 7 - RESOLUTION AND COMPONENT FLAGS
(GDS Octet 17)

Bit	Value	Meaning

1	0	Direction increments not given
		1	Direction increments given

	2	0	Earth assumed spherical with
			radius = 6367.47 km
		1	Earth assumed oblate spheroid with size
			as determined by IAU in 1965: 
			6378.160 km, 6356.775 km, f = 1/297.0

	3-4		reserved (set to 0)

	5	0	u- and v-components of vector quantities
			resolved relative to easterly and
			northerly directions
		1	u and v components of vector quantities
			resolved relative to the defined grid in
			the direction of increasing x and y 
			(or i and j) coordinates	respectively

	6-8		reserved (set to 0)

TABLE 8.  SCANNING MODE FLAG
(GDS Octet 28)

BIT	VALUE			MEANING

	1	0	Points scan in +i direction
		1	Points scan in -i direction

	2	0	Points scan in -j direction
		1	Points scan in +j direction

	3	0	Adjacent points in i direction are consecutive
			  (FORTRAN: (I,J))
		1    Adjacent points in j direction are consecutive
			  (FORTRAN: (J,I))

Note: 	i direction is defined as west to east along a parallel of latitude, or left to right along an x axis.

j direction is defined as south to north along a meridian of longitude, or bottom to top along a y axis.

TABLE 9. SPECTRAL REPRESENTATION TYPE
(GDS Octet 13)

VALUE 	      MEANING

	1	Associated Legendre Polynomials
		of the First Kind with normalization 
		such that the integral equals 1

TABLE 10. COEFFICIENT STORAGE MODE
(GDS Octet 14)

VALUE	         MEANING

1 	The complex coefficients Xnm are stored for   m > 0 as pairs of real numbers Re(Xnm), Im(Xnm) ordered with n increasing from m to N(m), first for m = 0 and then for m = 1, 2, 3,...M.  The real part of the (0,0) coefficient is stored in octets 12-15 of the BDS, as a floating point number in the same manner as the packing reference value, with units as in Table 2.  The remaining coefficients, starting with the imaginary part of the (0,0) coefficient, are packed according to the GRIB packing algorithm, with units as given in Table 5, in octets 16 and onward in the BDS.

NOTES ON SPECTRAL TRUNCATION:

Using the associated Legendre Polynomials of the First Kind, Pnm, as typical expansion functions, any variable x(l,m), which is a function of longitude, l , and sin(latitude), m , can be represented by

µ §

In the summations, M is the maximum zonal wave number that is to be included, and K & J together define the maximum meridional total wave number N(m), which, it should be noted, is a function of m.  A sketch shows the relationships:

µ §

In this figure, the ordinate is n the zonal wave number, the abscissa, m, is the total meridional wave number, the vertical line at m = M is the zonal truncation, and the diagonal passing through (0,0) is the line n = m.  The Legendre Polynomials are defined only on or above this line, that is for n > m.  On the n-axis, the horizontal line at n = K indicates the upper limit to n values, and the diagonal that intersects the n-axis at n = J indicates the upper limit of the area in which the Polynomials are defined.  The shaded irregular pentagon defined by the n-axis, the diagonal from n = J, the horizontal n = K, the vertical m = M, and the other diagonal n = m surrounds the region of the (n x m) plane containing the Legendre Polynomials used in the expansion.

This general pentagonal truncation reduces to some familiar common truncations as special cases:

Triangular: 	K = J = M and N(m) = J
Rhomboidal: 	K = J + M and N(m) = J + m
Trapezoidal:	K = J, K > M and N(m) = J

In all of the above m can take on negative values to represent the imaginary part of the spectral coefficients.

section 3: BIT MAP SECTION (BMS).

The purpose of the (optional) BMS is to provide either a bit map or a reference to a bit map pre-defined by the center.  The bit map consists of contiguous bits with a bit-to-data-point correspondence as defined in the grid description.  A bit set equal to 1 implies the presence of a datum for that grid point in the BDS; a value of zero implies the absence of such.

Octet no.

1 - 3       Length in octets of Bit Map Section

4           Number of unused bits at end of Section 3.

5 - 6       Numeric:

Numeric = 0: a bit map follows;
                    otherwise  : the numeric refers to a
						   predefined bit map provided 
						   by the center

7 - nnn     Bit map, zero filled to an even number of octets

section 4: BINARY DATA SECTION (BDS).

The BDS contains the packed data and the binary scaling information needed to reconstruct the original data from the packed data.  The required decimal scale factor is found in the PDS, above.  The data stream is zero filled to an even number of octets.

Octet no.

1 - 3       Length in octets of binary data section

4		   Bits 1 through 4: Flag - See Table 11
		        Bits 5 through 8: Number of unused bits at end
			                     of Section 4.

 	 5 - 6       The binary scale factor (E).  A negative value
			   is indicated by setting the high order bit
			   (bit No. 1) in octet 5 to 1 (on).

7 - 10      Reference value (minimum value); floating
			   point representation of the number.

11          Number of bits into which a datum point is
			   packed

12 -nnn     Variable, depending on octet 4;
			   zero filled to an even number of octets.

14	Optionally, may contain an extension of the flags in octet 4.  See Table 11.

Here are some of the various forms the binary data can take; the flag table in BDS octet 4, possibly extended into octet 14, identifies which variant is in use.

Grid-point data - Simple packing

Here the data simply begin in octet 12 and continue, packed according to the algorithm described above, without any particular regard for computer "word" boundaries, until there is no more data.  There may be some "zero-fill" bits at the end.

If all the data in a grid point field happen to have the same value, then all of the deviations from the reference value are set to zero.  Since a zero value requires no bits for packing, octet 11 is set to zero, thus indicating a field of constant data, the value of which is given by the reference value.  Under these circumstances, octet 12 is set to zero (the required "zero fill to an even number of octets") and bits 5-8 of octet 4 contain an 8.  The number of data points in the field is implied by the grid identification given in the PDS and/or the GDS and BMS.

Spherical Harmonic Coefficients - Simple packing

Octets 12-15 contain the real part of the (0.0) coefficient in the same floating point format as the reference value in octets 7-10.  The imaginary part of the (0.0) coefficient, mathematically, is always equal zero.  Octets 16 to the end contain the remaining coefficients packed up as binary data with the same sort of scaling, reference value, and the like, as with grid-point numbers.  Excluding the (0,0) coefficient, which is usually much larger than the others, from the packing operation means that the remaining coefficients can be packed to a given precision more efficiently (fewer bits per word) than would be the case otherwise.

Grid-Point Data - Complex Packing

[This section to be completed at a later date.]

TABLES FOR THE BDS

TABLE 11.  FLAG
(BDS Octet 4 and, optionally, 14)

Bit     Value	   Meaning

1	0	Grid point data	
		1	Spherical Harmonic Coefficients

	2	0	Simple packing 
		1	Second order ("Complex") Packing 

 	3	0	Original data were floating point values
		1	Original data were integer values

	4	0	No additional flags at octet 14
		1	Octet 14 contains flag bits 5 - 12
			
The following gives the meaning of the bits in octet 14 ONLY if bit 4 is set to 1.  Otherwise octet 14 contains regular binary data.  

	5		Reserved (set to 0)

6	0	Single datum at each grid point
		1	Matrix of values at each grid point

	7	0	No secondary bit maps
		1	Secondary bit maps present

	8	0	Second order values have constant width
		1	Second order values have different widths

	9-12		Reserved (set to 0)

Notes:

(1)	Bit 4 is set to 1 to indicate that bits 5 to 12 are contained in octet 14 of the data section.

(2)	Bit 3 is set to 1 to indicate that the original data was integers; when this is the case any non-zero reference values should be rounded to an integer value.

(3)	When secondary bit maps are present in the data (used in association with second order packing and, optionally, with a matrix of values at each point) this is indicated by setting bit 7 to 1.

(4)	When octet 14 contains the extended flag information octets 12 and 13 will also contain "special" information; the actual data will begin in a subsequent octet.  See above.

At present, the "extension" of Table 11 into octet 14 and the associated "advanced" features of GRIB are limited to spherical harmonics and second order("complex") packing in the National Weather Service.  Additional variations are included in the WMO Documentation.

section 5: End Section

The end section serves a human readable indication of the end of a GRIB record.  It can also be used for computer verification that a complete GRIB record is available for data extraction.  It should not be used as a search target since a '7777' bit combination could exist anywhere in the binary data stream.

Octet no.

1-4		  '7777' (Coded CCITT-ITA No. 5)

APPENDIX A

OUTLINE OF WMO BULLETIN HEADERS

USED WITH

G  R  I  B

WMO BULLETIN HEADER

The WMO abbreviated heading is used to identify the NMC GRIB messages; however, it is not a complete description of their content.  The user is cautioned against using the header as the sole determiner of the record content; she should, of course, rely on the Product Definition Section for that purpose.

Note: In the following, a hexadecimal number is enclosed in parentheses followed by the designation "hex".

The information needed to identify the NMC product is contained in 21 octets.  The characters are encoded using the CCITT-ITA No. 5, also known (in the US) as ASCII characters, and are defined as follows:

Octet no.				Header Content

1     The character 'H' for GRIB bulletins sent to the NWS Family of Services, used for the WAFS program, and for general International Exchange
or
The characters 'Y' or 'Z' for GRIB bulletins intended for the NWS AWIPS program.

2     A letter character specifying the type parameter as shown in Table A.1.

3     A letter character specifying the grid area as defined in Table A.2.

4     A letter or numeric character indicating the time difference between the reference time and valid time of the data as listed in Table A.3, i.e., the forecast length.

5-6   Numeric characters as defined in Table A.4.  Usually the pressure level, sometimes just a sequence number.  Some values have special level or layer meanings.

7     Blank (20)hex

8-11  Four characters identifying the originating center.  These are always 'KWBC' for NMC-produced messages.

12    Blank (20)hex

13-14 Two numeric characters providing the reference day of the month (01-31) of the data.

15-18 Four numeric characters providing the reference hour and minute of the data.

19-22 Four OPTIONAL characters: one blank (20)hex, then 'Pxx', where xx=01-99.  Used to indicate sequential parts of very long messages that have been divided using the BLOK feature. See Appendix B.

19-21 OR 23-25  Two ASCII carriage returns and a line feed, (0D0D0A)hex.

The first six characters are commonly referred to as

T1 T2 A1 A2 ii

In summary...

Generic Meaning of   T1  T2  A1  A2  ii:

T1:	Type of bulletin:  	"H" for GRIB messages for
						Family of Services, WAFS, and
						International Exchange;
						"Y" or "Z" for AWIPS GRIB messages

T2:	Type of data/parameter

A1:	Grid

A2:	Analysis or forecast hour

ii:	Numeric.  Usually the pressure level, sometimes just a sequence number.  Some values have special level or layer meanings.

In the following tables, the columns headed AWIPS are augmentations to the common Family of Services (FOS),National, and International Exchange variables.  FOS, National and International GRIB messages (with H as the initial character) draw upon the left hand columns only.  AWIPS GRIB messages (with Y or Z as the initial character) use letters from both columns.  If both columns contain entries for the same designator, the T1 character (H, Y, or Z) indicates which entry to use.

TABLE A.1   TYPE PARAMETERS - T2
(Header Octet 2)

DESIGNATOR	             PARAMETER

Usage

	   FOS & International (H)			AWIPS (Y or Z)

A
B	Vertical Wind Shear
C	Vorticity
D	Thunderstorm probability
E       Total Precipitation            	
F       Long Wave Radiation	Precipitable Water
G	Convective Precip.
H       Height (geopotential)    
I
J
K       Primary Wave Period      
L       Primary Wave Direction   
M       Secondary Wave Period
N       Secondary Wave Direction
O       Vertical velocity        
P       Pressure                 
Q	Stability Index
	(Best 4-layer index)
R       Relative humidity        
S       Snow                     
T       Temperature              
U       u wind component  
V       v wind component 
W
X       Surface Lifted index
Y
Z

TABLE A.2   GRID DESIGNATOR - A1
(Header Octet 3)

DESIGNATOR	        GRID Number 
		       (See Table B)

	  FOS & International (H) 		AWIPS (Y or Z)

	A	21	201 - Northern Hemisphere
	B	22
	C	23
	D	24
	E	25
	F	26
	G	50
	H		213 - National CONUS with
			      Double Resolution
	I	37	202 - National CONUS
	J	38	203 - National Alaska
	K	39	204 - National Hawaii
	L	40	205 - National Puerto Rico
	M	41	206 - Regional MARD
	N	42	207 - Regional Alaska
	O	43	208 - Regional Hawaii
	P	44	210 - Regional Puerto Rico
	Q		211 - Regional CONUS
	R		212 - Regional CONUS with
				    Double Resolution
	S		209 - Regional MARD with
				    Double Resolution
	T	61	214 - Regional Alaska with
				    Double Resolution
	U	62
	V	63
	W	64
	X        (Used for experimental transmissions)
	Y
	Z

TABLE A.3   FORECAST HOUR DESIGNATOR - A2
(Header Octet 4)

DESIGNATOR       HOUR

FOS & International (H)

    A       00 hour analysis
    B       06 hour fcst
    C       12     "
    D       18     "
    E       24     "
    F       30     "
    G       36     "
    H       42     "
    I       48     "
    J       60     "
    K       72     "
    L       84     "
    M       96     "
    N      108     "
    O      120     "
    P      132     "
    Q      144     "
    R      156     "
    S      168     "
    T      180     "
    U      192     "
    V      204     "
    W      216     "
    X      228     "
    Y      240     "
    Z      Reserved for special purposes

Note: The following designators are used for AWIPS only, with "Z" as the first character in the header.

DESIGNATOR	HOUR

A	2 hour fcst
	B	3	"
	C	4	"
	D	8	"
	E	9	"
	F	10	"
	G	14	"
	H	15	"
	I	16	"
	J	20	"
	K	21	"
	M	54	"
	N	66	"

TABLE A.4   LEVEL DESIGNATORS - ii
(Header Octets 5 and 6)
(H, Y, or Z)

DESIGNATOR		LEVEL or LAYER

00	Entire Atmosphere
99	1000 hPa
98	Air Properties at Surface of Earth
97	Level of the tropopause
96	Level of the maximum wind
95	950 hPa
94	Level of 0 deg C isotherm
93	Land/Water Properties at Surface of
	Earth/Ocean
92	Boundary Layer
91	Any parameter reduced to Sea Level

Note: The following levels are used to indicate geometric height for aviation flight levels, not pressure levels

81	810 hPa ~ 1828 m = 6000 ft FL
73	730 hPa ~ 2743 m = 9000 ft FL
64	640 hPa ~ 3658 m = 12000 ft FL

Otherwise, the designator given is the hundreds and tens digits of the hPa level in the atmosphere, e.g. 70=700hPa; 03=30hPa, etc..

SPECIAL NOTE

The following version of Table A.4 contains changes recently approved by the WMO.  They are NOT in effect as yet but are scheduled to go into effect on November 3, 1993.  The table is presented here for your edification, so you can make proper preparations, so I won't have to print a new copy of this document after Nov. 3, and so I can say "I told you so!" when all your codes fail on Nov. 4, 1993

TABLE A.4   LEVEL DESIGNATORS - ii
(Header Octets 5 and 6)
(H, Y, or Z)

DESIGNATOR        LEVEL or LAYER

00	Entire Atmosphere
99	1000 hPa
98	Air Properties at Surface of Earth
97	Level of the tropopause
96	Level of the maximum wind
95	950 hPa
94	Level of 0 deg. C isotherm
93	Not assigned
92	925 hPa 
91	Not assigned
90	900 hPa
89	Any parameter reduced to Sea Level
88	Land/Water Properties at Surface of
	Earth/Ocean
87	Not assigned
86	Boundary Layer

Note: The following levels are used to indicate geometric height for aviation flight levels, not pressure levels

81	810 hPa ~ 1828 m = 6000 ft FL
73	730 hPa ~ 2743 m = 9000 ft FL
64	640 hPa ~ 3658 m = 12000 ft FL

Otherwise, the designator given is the hundreds and tens digits of the hPa level in the atmosphere, e.g. 70=700hPa; 03=30hPa, etc..

APPENDIX B

The BLOK feature

for subdividing large GRIB (and/or BUFR) bulletins

BLOK

The BLOK feature is used to subdivide large GRIB bulletins to allow them to fit into existing (limited) communications or data storage facilities.  Once divided, the individual subdivisions (BLOKs) can not stand alone - they must be reassembled back into the single bulletin whence they came for any processing.  However, from a communications or storage standpoint, each individual BLOK is considered as a distinct bulletin or record and will be uniquely identified.

The application of the BLOK feature is quite straightforward: a given GRIB record - all the bits from "GRIB" through "7777" - may be partitioned at any point (without any regard for the contents) or any number of points.  The only rule is that the separate parts must contain an even number of octets.  The separate parts are then encased in a BLOK envelope (details follow), and that's it.  It is, of course, up to the receiver of a collection of BLOKs to strip off the BLOK envelopes and put the GRIB message back together again.  Just get all the king's horses and all the king's men to help do so.

Here is the structure and content of the BLOK envelope.

Section 0 - Indicator section

Octet No.	Contents

1-4	'BLOK' - 4 ASCII characters
	5-7	Total length of this BLOK, in octets
	 8	BLOK Edition Number - currently 0
	9-11	Total length of original GRIB message, 
		in octets
	12	Flag - see Table B.1 - indicates presence 
		or absence of Section 1
	13	Total number of related BLOKs
	14	Sequence number of this BLOK
	15-16	reserved, set to 0

Section 1 - Identification section - optional

If present, this section contains the complete Product Definition Section (PDS) from the original GRIB message.  If this section is included in one BLOK, it must be included in all related BLOKs.

Section 2 - Data section

This section contains an arbitrary part of the original GRIB message - the only restriction is that this section contain an even number of octets.

Section 3 - End section

Octet No.	Contents

1-4	'7777' - 4 ASCII characters

Table B.1

Section 1 present Flag

Bit	Value	Meaning

1	0	Optional Section 1 omitted
		1	Optional Section 1 present

	2-8		Reserved (set to 0)

Keep in mind that each subdivision of the original bulletin will be completely surrounded by a BLOK envelope, from 'BLOK' through '7777'.

Note that the last BLOK of a set for a given bulletin will have two '7777's in a row; the first is the one that marks the end of the original bulletin, the second marks the end of the (last) BLOK.

When BLOKs are sent out as individual messages with identifying WMO headers, the 'Pxx' characters that are an optional part of the header shall be included in the header.  See Appendix A.  In this way the complete WMO header shall be unique for each message even though the TTAAii portion of the headers will be the same, reflecting that each BLOK, for a single GRIB message,  contains the same information.

<?>	World Meteorological Organization publication No. 306, Manual on Codes, Vol. 1, Part B, Secretariat of the WMO, Geneva, Switzerland, 1988, plus Supplements No. 1, 2, & 3